#define BLYNK_TEMPLATE_ID "TMPL3XYZU_KPQ"

#define BLYNK_TEMPLATE_NAME "Smart toilet monitoring system"

Y_CQSz8WsUtHX0Uk1KcrG5jDCHlefdwS

#define BLYNK_TEMPLATE_ID "TMPL3XYZU_KPQ"

#define BLYNK_TEMPLATE_NAME "Smart toilet monitoring system"

#define BLYNK_AUTH_TOKEN "Y_CQSz8WsUtHX0Uk1KcrG5jDCHlefdwS"

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <DHT.h>

// WiFi

char ssid[] = "pooja@123";

char pass[] = "12345678";

// OLED

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// IR

#define IR_A 14

#define IR_B 33

// DHT

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

// MQ135

#define MQ135_PIN 34

// Alert

#define BUZZER 13

#define RED_LED 12

int count = 0;

bool A_detected = false;

bool lockState = false;

unsigned long lastTempRead = 0;

float temp = 0;

unsigned long lastBlink = 0;

bool blinkState = false;

bool alertSent = false;

BlynkTimer timer;

// 🚶 FAST IR COUNT (A → B ONLY)

void handlePeopleCount() {

bool A = digitalRead(IR_A);

bool B = digitalRead(IR_B);

if (lockState) {

if (A == HIGH && B == HIGH) {

lockState = false;

A_detected = false;

}

return;

}

if (A == LOW && !A_detected) {

A_detected = true;

}

if (B == LOW && !A_detected) {

lockState = true;

}

if (A_detected && B == LOW) {

count++;

lockState = true;

}

}

// 🌡️ NON-BLOCKING TEMP

void readTemp() {

if (millis() - lastTempRead > 1000) {

float t = dht.readTemperature();

if (!isnan(t)) temp = t - 1.5;

lastTempRead = millis();

}

}

// 🔁 MAIN FUNCTION (SLOW TASKS)

void sendData() {

readTemp();

int gasValue = analogRead(MQ135_PIN);

bool alert = false;

String reason = "NORMAL";

if (temp > 33) {

alert = true;

reason = "HIGH TEMP";

}

else if (gasValue > 800) {

alert = true;

reason = "BAD SMELL";

}

else if (count > 10) {

alert = true;

reason = "OVER CROWD";

}

// 🔁 BLINK ALERT

if (alert) {

if (millis() - lastBlink > 300) {

blinkState = !blinkState;

digitalWrite(BUZZER, blinkState);

digitalWrite(RED_LED, blinkState);

lastBlink = millis();

}

} else {

digitalWrite(BUZZER, LOW);

digitalWrite(RED_LED, LOW);

}

// 📱 BLYNK

Blynk.virtualWrite(V0, temp);

Blynk.virtualWrite(V1, gasValue);

Blynk.virtualWrite(V2, count);

Blynk.virtualWrite(V3, reason);

Blynk.virtualWrite(V4, alert);

if (alert && !alertSent) {

Blynk.logEvent("toilet_alert", reason);

alertSent = true;

}

if (!alert) alertSent = false;

// 📺 OLED UI

display.clearDisplay();

display.setTextSize(1);

display.setCursor(0, 0);

display.print("Temp: "); display.print(temp);

display.setCursor(0, 10);

display.print("Gas: "); display.print(gasValue);

display.setCursor(0, 20);

display.print("Count: "); display.print(count);

display.setCursor(0, 35);

display.print("Status: "); display.print(reason);

if (alert) {

display.setCursor(0, 50);

display.print("Need to Clean!");

}

display.display();

}

// ⚙️ SETUP

void setup() {

Serial.begin(115200);

pinMode(IR_A, INPUT_PULLUP);

pinMode(IR_B, INPUT_PULLUP);

pinMode(BUZZER, OUTPUT);

pinMode(RED_LED, OUTPUT);

dht.begin();

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay();

display.setTextColor(WHITE);

// 🎉 WELCOME SCREEN

display.setTextSize(1);

display.setCursor(0, 20);

display.print("Welcome to Smart");

display.setCursor(0, 30);

display.print("Restroom!");

display.setCursor(0, 45);

display.print("Keep it Clean :)");

display.display();

delay(3000);

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

timer.setInterval(1000L, sendData);

}

// 🔁 LOOP (FAST TASKS)

void loop() {

handlePeopleCount();   // ⚡ ultra fast counting

Blynk.run();

timer.run();

}